# (2023Apr GDX.docx)

# RECOMMENDED ENGINEERING SPECIFICATION FOR GUIDELINE PIPE/TUBE ALIGNMENT GUIDES

## PART 1 GENERAL

1.01	SECTION INCLUDES

- Furnish and install pipe / tube alignment guides.

  - Model GDXP for use with pipe

  - Model GDXC including dielectric spacer (for use with copper tube)

1.02	MANUFACTURER

A.  Provide appropriate model pipe / tube alignment guides as supplied by Flex-Hose Co., or approved equal.

PART 2 PRODUCTS

2.01 Pipe alignment guides

- The guide(s) must allow movement along the pipe axis while restricting lateral pipe movement; thereby allowing proper operation of the expansion control device.

- The pipe alignment guide is to consist of:

  - Two piece spider

  - Two piece housing

  - Required hardware to assemble the spiders and the housings.

- The alignment guide housing and spider is to be constructed of carbon steel (painted for rust protection)

- The guide design must provide a bolted split-spider clamp/slide and a bolted split-housing with integral base for anchoring the guide by bolting or welding (guides are to be shipped unassembled for field installation).

- If used on copper tubing piping systems lead dielectric spacer to be provided for use between copper tube and spider

  - WARRANTY

Pipe alignment guides are to be warrantied for a period of two years when installed in accordance with all specifications and instructions of the manufacturer and compliant with EJMA (Expansion Joint Manufacturers Association) spacing requirements.